HomeSense Node - CO2 Monitor Firmware (Week 2)

Source file: homesense_firmware.c  |  Author: Mangesh Biradar  |  Yuva Intern - Embedded Systems

/*

* =============================================================================

*  HomeSense Node - CO2 Monitor Firmware (Week 2: Firmware Development)

*  File    : homesense_firmware.c      Version : 1.0

*  Author  : Mangesh Biradar             Program : Yuva Intern - Embedded Systems

* =============================================================================

*

*  README

*  ------

*  1. WHAT IT DOES

*     A small firmware slice of the HomeSense Node air-quality monitor designed

*     in Week 1. It reads a CO2 value, filters it, decides whether the air is

*     GOOD / FAIR / POOR, shows the result on three LEDs, sounds a buzzer when

*     the air is poor, and talks to the user over UART (serial) commands.

*

*  2. FEATURES (and the embedded concept each one demonstrates)

*     - 1 ms hardware TIMER INTERRUPT      -> time base and cooperative scheduler

*     - UART RX INTERRUPT + ring buffer    -> safe interrupt-to-main data transfer

*     - UART command line (HELP, STATUS..) -> command parser, serial debugging

*     - ADC sensor read + conversion       -> sensor data reading (potentiometer)

*     - 4-sample moving-average filter     -> noise reduction

*     - Alert state machine                -> persistence (3 samples) + hysteresis

*     - Push-button with 30 ms debounce    -> mutes the buzzer

*     - Sensor-fault detection             -> fault-tolerant behaviour

*     - Hardware Abstraction Layer (HAL)   -> same logic runs on PC and on AVR

*

*  3. HOW THE CODE IS ORGANISED (single file, top to bottom)

*       Section 1  Configuration constants

*       Section 2  HAL interface (functions the core needs from the hardware)

*       Section 3  Core firmware: ISR handlers, UART, commands, sensor logic,

*                  outputs, button, app_init(), app_step()

*       Section 4  HAL for real hardware: ATmega328P / Arduino Uno (avr-gcc)

*       Section 5  HAL for PC simulation + automated self-tests (gcc)

*

*     Execution model ("foreground / background"):

*       Interrupts (background)         Main loop (foreground)

*       timer_tick_isr()  every 1 ms -> sets flags 10 ms / 250 ms / 1 s

*       uart_rx_isr()     per byte    -> pushes byte into ring buffer

*                                       app_step(): reads flags + buffer and

*                                       runs button_task / blink / sample_task

*     ISRs stay tiny; all real work happens in app_step().

*

*  4. HOW TO BUILD AND RUN

*     A) PC SIMULATION (no hardware needed) - tested for this submission

*          gcc -std=c99 -Wall -Wextra -pedantic -DHOST_SIM \

*              -o homesense_sim homesense_firmware.c

*          ./homesense_sim                 run the 12 automated tests

*          ./homesense_sim --trace         same, plus LED/buzzer changes

*          ./homesense_sim --interactive   type commands yourself:

*              any text      -> sent to the device as a UART command

*              @run 3000     -> advance simulated time by 3000 ms

*              @adc 700      -> set the simulated potentiometer (0..1023)

*              @btn 1 / @btn 0 -> press / release the button

*              @quit         -> exit

*     B) REAL HARDWARE / SIMULATOR (ATmega328P, e.g. Arduino Uno at 16 MHz)

*          avr-gcc -mmcu=atmega328p -DF_CPU=16000000UL -Os -Wall -Wextra \

*                  -o homesense.elf homesense_firmware.c

*          avr-objcopy -O ihex -R .eeprom homesense.elf homesense.hex

*          simulate:  simavr -m atmega328p -f 16000000 homesense.elf

*          or upload homesense.hex with avrdude / a simulator that loads .hex.

*        Wiring (Arduino Uno):

*          D8  -> green LED  -> 220 ohm -> GND     D9  -> yellow LED (same)

*          D10 -> red LED (same)                   D11 -> active buzzer -> GND

*          D2  -> push button -> GND (internal pull-up used)

*          A0  <- potentiometer wiper (ends to 5V and GND) = simulated CO2 sensor

*          Serial monitor: 9600 baud, 8N1, line ending "Newline" or "Both".

*

*  5. UART COMMANDS (not case sensitive, end with Enter)

*       HELP            list commands

*       STATUS          print CO2, level, alert, threshold, mute, source, fault

*       TH <ppm>        set alert threshold, 1100..5000 (default 1500)

*       INT <s>         set report interval in seconds, 1..60 (default 5)

*       MUTE / UNMUTE   disable / enable the buzzer

*       SIM <ppm>       test mode: use a fixed CO2 value (400..5000)

*       SIM FAIL        test mode: pretend the sensor is broken

*       SIM ADC         back to the real ADC input

*

*  6. BEHAVIOUR RULES

*       CO2 ppm = 400 + ADC * 1600 / 1023           (ADC 0 -> 400, 1023 -> 2000)

*       GOOD  : filtered CO2 below 1000 ppm        -> green LED solid

*       FAIR  : 1000 ppm up to the threshold       -> yellow LED solid

*       POOR  : alert active                       -> red LED blinks 2 Hz + buzzer

*       Alert ON  after 3 consecutive samples at or above the threshold.

*       Alert OFF when the filtered value falls 50 ppm below the threshold.

*       FAULT : 3 failed reads in a row            -> all LEDs blink together.

*

*  7. KEY FUNCTIONS

*       timer_tick_isr()  1 ms tick, produces the 10 ms/250 ms/1 s flags

*       uart_rx_isr()     stores a received byte in the ring buffer

*       app_step()        one pass of the main loop (the scheduler)

*       sample_task()     read -> filter -> alert logic -> LEDs -> report

*       alert_eval()      persistence + hysteresis state machine

*       outputs_apply()   decides LED pattern and buzzer from the state

*       button_task()     30 ms debounce, press toggles mute

*       cmd_exec()        parses and runs one UART command line

*

*  8. TEST RESULTS (PC simulation, see Section 5)

*       Environment: gcc 13 on Linux, built with -std=c99 -Wall -Wextra -pedantic

*       (zero warnings); also run clean under AddressSanitizer + UBSan.

*       Result: 45 of 45 checks passed across 12 test groups:

*         T0 boot banner          T1 timer 1 ms time base   T2 ADC->ppm maths

*         T3 GOOD level + report  T4 FAIR level             T5 alert persistence

*         T6 MUTE/UNMUTE          T7 button debounce        T8 hysteresis

*         T9 UART commands        T10 SIM override          T11 sensor fault

*         T12 UART buffer overflow

*       Test-quality check: 4 deliberate bugs were injected (persistence 3->1,

*       hysteresis 50->0, debounce removed, wrong ADC scale); each was caught

*       by at least one failing test, so the tests are not passing vacuously.

*       Hardware (AVR) back-end: written from the ATmega328P datasheet register

*       usage; build and run it in a simulator as described in section 4B.

*

*  9. KNOWN LIMITS / FUTURE WORK

*       Mute does not time out; CO2 comes from a potentiometer, not a real

*       SCD41; no flash storage or Wi-Fi yet (see Week 1 design).

* =============================================================================

*/

#include <stdint.h>

#include <stdbool.h>

#include <string.h>

#if defined(__AVR__)

#define TARGET_AVR 1

#ifndef F_CPU

#define F_CPU 16000000UL

#endif

#include <avr/io.h>

#include <avr/interrupt.h>

#include <avr/sleep.h>

#elif defined(HOST_SIM)

#include <stdio.h>

#include <stdlib.h>

#else

#error "Build with -DHOST_SIM for the PC simulation, or with avr-gcc -mmcu=atmega328p"

#endif

/* =============================================================================

* SECTION 1 - CONFIGURATION

* ========================================================================== */

#define FW_VERSION        "1.0"

#define FILTER_LEN        4      /* moving-average window (samples)            */

#define FAIR_PPM          1000   /* at or above this the air is "FAIR"         */

#define TH_DEFAULT        1500   /* default alert threshold (ppm)              */

#define TH_MIN            1100   /* allowed range for the TH command           */

#define TH_MAX            5000

#define HYST_PPM          50     /* alert clears this far below the threshold  */

#define PERSIST_SAMPLES   3      /* samples above threshold before alert       */

#define FAULT_LIMIT       3      /* failed reads in a row before FAULT         */

#define REPORT_DEFAULT_S  5      /* seconds between periodic reports           */

#define DEBOUNCE_TICKS    3      /* 3 x 10 ms = 30 ms stable before accepting  */

#define RX_SIZE           64     /* UART ring buffer size, MUST be power of 2  */

#define LINE_MAX_LEN      24     /* longest accepted command line (chars)      */

#define LED_G 0x01u              /* bit masks used by hal_led_set()            */

#define LED_Y 0x02u

#define LED_R 0x04u

typedef enum { LVL_GOOD = 0, LVL_FAIR, LVL_POOR } level_t;

typedef enum { SRC_ADC = 0, SRC_PPM, SRC_FAIL } source_t;

/* =============================================================================

* SECTION 2 - HAL INTERFACE (implemented in Sections 4 and 5)

* ========================================================================== */

static void     hal_init(void);                 /* configure pins/timer/UART/ADC */

static void     hal_led_set(uint8_t mask);      /* LED_G | LED_Y | LED_R         */

static void     hal_buzzer(bool on);

static int16_t  hal_adc_read(void);             /* 0..1023, or -1 on failure     */

static bool     hal_button_pressed(void);       /* true while button is down     */

static void     hal_uart_putc(char c);          /* blocking transmit of 1 byte   */

static uint8_t  hal_irq_save(void);             /* disable interrupts, old state */

static void     hal_irq_restore(uint8_t state);

/* =============================================================================

* SECTION 3 - CORE FIRMWARE

* ========================================================================== */

/* ---- 3.1 Interrupt-shared data ------------------------------------------ */

/* Flags are set in the ISR and cleared in app_step(). 'volatile' tells the

* compiler they can change at any time. 8-bit accesses are atomic on AVR.   */

static volatile uint8_t f_10ms, f_250ms, f_1s;

static uint16_t c_10, c_250, c_1000;            /* ISR-private down-counters    */

/* Called by the hardware timer every 1 ms. Keeps work to an absolute minimum:

* only counts and raises flags. If the main loop is slow, flags simply merge. */

void timer_tick_isr(void)

{

if (++c_10   >= 10)   { c_10   = 0; f_10ms  = 1; }

if (++c_250  >= 250)  { c_250  = 0; f_250ms = 1; }

if (++c_1000 >= 1000) { c_1000 = 0; f_1s    = 1; }

}

/* UART receive ring buffer. The ISR is the only writer of rx_head, the main

* loop is the only writer of rx_tail, so no lock is needed.                  */

static volatile uint8_t rx_buf[RX_SIZE];

static volatile uint8_t rx_head, rx_tail;

static volatile uint8_t rx_overflow;            /* bytes dropped when full      */

/* Called by the hardware on every received byte. */

void uart_rx_isr(uint8_t c)

{

uint8_t next = (uint8_t)((rx_head + 1u) & (RX_SIZE - 1u));

if (next == rx_tail) {                      /* buffer full: drop the byte   */

if (rx_overflow < 255u) rx_overflow++;

return;

}

rx_buf[rx_head] = c;

rx_head = next;

}

static bool rx_get(uint8_t *c)

{

if (rx_head == rx_tail) return false;

*c = rx_buf[rx_tail];

rx_tail = (uint8_t)((rx_tail + 1u) & (RX_SIZE - 1u));

return true;

}

/* ---- 3.2 Application state (main-loop only) ---------------------------- */

static uint32_t s_uptime;                       /* seconds since start          */

static uint16_t s_fbuf[FILTER_LEN];             /* moving-average history       */

static uint8_t  s_fcnt, s_fidx;

static uint16_t s_inst = 400;                   /* latest instantaneous ppm     */

static uint16_t s_filt = 400;                   /* filtered ppm                 */

static level_t  s_level = LVL_GOOD;

static bool     s_alert;

static uint8_t  s_persist;

static uint16_t s_th = TH_DEFAULT;

static uint8_t  s_report_s = REPORT_DEFAULT_S;

static uint8_t  s_rep_cnt;

static bool     s_muted;

static bool     s_blink;                        /* toggles every 250 ms         */

static bool     s_fault;

static uint8_t  s_fault_cnt;

static source_t s_src = SRC_ADC;

static uint16_t s_sim_ppm;

static bool     s_btn_raw, s_btn_stable;        /* debounce state               */

static uint8_t  s_btn_cnt;

static char     s_line[LINE_MAX_LEN + 1];       /* command line being typed     */

static uint8_t  s_line_len;

static bool     s_line_discard;                 /* skip rest of an overlong line*/

/* ---- 3.3 Small output helpers (no printf, so it fits a small MCU) ------- */

static void put_str(const char *s) { while (*s) hal_uart_putc(*s++); }

static void put_nl(void)           { hal_uart_putc('\r'); hal_uart_putc('\n'); }

static void put_u32(uint32_t v)

{

char b[11];

uint8_t i = 0;

if (v == 0) { hal_uart_putc('0'); return; }

while (v) { b[i++] = (char)('0' + (v % 10u)); v /= 10u; }

while (i)  hal_uart_putc(b[--i]);

}

static void put_ts(void)                        /* "[t=12s] " time stamp        */

{

put_str("[t="); put_u32(s_uptime); put_str("s] ");

}

static const char *level_name(level_t l)

{

return (l == LVL_GOOD) ? "GOOD" : (l == LVL_FAIR) ? "FAIR" : "POOR";

}

static const char *src_name(source_t s)

{

return (s == SRC_ADC) ? "ADC" : (s == SRC_PPM) ? "PPM" : "FAIL";

}

/* Print "[t=..s] EVT <msg> CO2=..ppm" */

static void event_ppm(const char *msg)

{

put_ts(); put_str("EVT "); put_str(msg);

put_str(" CO2="); put_u32(s_filt); put_str("ppm"); put_nl();

}

/* ---- 3.4 Sensor conversion and filter ----------------------------------- */

/* Linear map of the 10-bit ADC value onto 400..2000 ppm.                     */

static uint16_t adc_to_ppm(uint16_t adc)

{

return (uint16_t)(400u + ((uint32_t)adc * 1600u) / 1023u);

}

static void filter_push(uint16_t v)

{

uint32_t sum = 0;

uint8_t i;

s_fbuf[s_fidx] = v;

s_fidx = (uint8_t)((s_fidx + 1u) % FILTER_LEN);

if (s_fcnt < FILTER_LEN) s_fcnt++;

for (i = 0; i < s_fcnt; i++) sum += s_fbuf[i];

s_filt = (uint16_t)(sum / s_fcnt);

}

/* ---- 3.5 Alert state machine (persistence + hysteresis) ----------------- */

/*   normal --(filt >= TH for 3 samples)--> ALERT

*   ALERT  --(filt + 50 < TH)-----------> normal                             */

static void alert_eval(void)

{

if (!s_alert) {

if (s_filt >= s_th) {

if (++s_persist >= PERSIST_SAMPLES) {

s_alert = true;

s_persist = 0;

event_ppm("ALERT ON");

}

} else {

s_persist = 0;                      /* streak broken                */

}

} else if ((uint16_t)(s_filt + HYST_PPM) < s_th) {

s_alert = false;

event_ppm("ALERT OFF");

}

if (s_alert)                    s_level = LVL_POOR;

else if (s_filt >= FAIR_PPM)    s_level = LVL_FAIR;

else                            s_level = LVL_GOOD;

}

/* ---- 3.6 LED and buzzer outputs ----------------------------------------- */

static void outputs_apply(void)

{

uint8_t mask = 0;

bool buz = false;

if (s_fault) {                              /* all LEDs blink together      */

mask = s_blink ? (uint8_t)(LED_G | LED_Y | LED_R) : 0u;

} else if (s_level == LVL_GOOD) {

mask = LED_G;

} else if (s_level == LVL_FAIR) {

mask = LED_Y;

} else {                                    /* POOR: red blink + beep       */

mask = s_blink ? LED_R : 0u;

buz  = s_blink && !s_muted;

}

hal_led_set(mask);

hal_buzzer(buz);

}

/* ---- 3.7 Sampling task (runs once per second) --------------------------- */

static void sample_task(void)

{

bool ok = true;

uint16_t ppm = 0;

s_uptime++;

if (s_src == SRC_FAIL) {

ok = false;                             /* injected failure (SIM FAIL)  */

} else if (s_src == SRC_PPM) {

ppm = s_sim_ppm;                        /* injected value (SIM <ppm>)   */

} else {

int16_t raw = hal_adc_read();

if (raw < 0 || raw > 1023) ok = false;  /* plausibility check           */

else ppm = adc_to_ppm((uint16_t)raw);

}

if (!ok) {

if (s_fault_cnt < 255u) s_fault_cnt++;

if (s_fault_cnt >= FAULT_LIMIT && !s_fault) {

s_fault = true;

event_ppm("SENSOR FAULT");

}

} else {

if (s_fault) { s_fault = false; event_ppm("SENSOR OK"); }

s_fault_cnt = 0;

s_inst = ppm;

filter_push(ppm);

alert_eval();

}

outputs_apply();

if (++s_rep_cnt >= s_report_s) {            /* periodic report              */

s_rep_cnt = 0;

put_ts();

if (s_fault) {

put_str("RPT FAULT");

} else {

put_str("RPT CO2="); put_u32(s_filt);

put_str("ppm inst="); put_u32(s_inst);

put_str(" LVL="); put_str(level_name(s_level));

}

put_nl();

}

}

/* ---- 3.8 Button task (runs every 10 ms) --------------------------------- */

/* The raw level must stay the same for DEBOUNCE_TICKS reads in a row before it

* is accepted, which removes contact bounce. A accepted PRESS toggles mute.  */

static void button_task(void)

{

bool raw = hal_button_pressed();

if (raw == s_btn_raw) {

if (s_btn_cnt < 255u) s_btn_cnt++;

} else {

s_btn_raw = raw;

s_btn_cnt = 0;

}

if (s_btn_cnt == DEBOUNCE_TICKS && s_btn_raw != s_btn_stable) {

s_btn_stable = s_btn_raw;

if (s_btn_stable) {                     /* press edge                   */

s_muted = !s_muted;

put_ts(); put_str("EVT BUTTON MUTE="); put_u32(s_muted ? 1u : 0u);

put_nl();

outputs_apply();

}

}

}

/* ---- 3.9 UART command interpreter --------------------------------------- */

static bool parse_u32(const char *s, uint32_t *out)

{

uint32_t v = 0;

if (*s == '\0') return false;

while (*s) {

if (*s < '0' || *s > '9') return false;

v = v * 10u + (uint32_t)(*s - '0');

if (v > 100000u) return false;

s++;

}

*out = v;

return true;

}

static void cmd_status(void)

{

put_str("CO2="); put_u32(s_filt);

put_str("ppm LVL="); put_str(level_name(s_level));

put_str(" ALERT="); put_u32(s_alert ? 1u : 0u);

put_str(" TH="); put_u32(s_th);

put_str(" INT="); put_u32(s_report_s);

put_str("s MUTE="); put_u32(s_muted ? 1u : 0u);

put_str(" SRC="); put_str(src_name(s_src));

put_str(" FAULT="); put_u32(s_fault ? 1u : 0u);

put_nl();

}

static void cmd_exec(char *line)

{

char *arg = line;

uint32_t n;

uint8_t i;

for (i = 0; line[i]; i++)                   /* upper-case: not case sensitive*/

if (line[i] >= 'a' && line[i] <= 'z') line[i] = (char)(line[i] - 32);

while (*arg && *arg != ' ') arg++;          /* split "CMD ARG"              */

if (*arg) { *arg++ = '\0'; while (*arg == ' ') arg++; }

if (!strcmp(line, "HELP")) {

put_str("CMDS: HELP STATUS TH <ppm> INT <s> MUTE UNMUTE SIM <ppm|ADC|FAIL>");

put_nl();

} else if (!strcmp(line, "STATUS")) {

cmd_status();

} else if (!strcmp(line, "TH")) {

if (parse_u32(arg, &n) && n >= TH_MIN && n <= TH_MAX) {

s_th = (uint16_t)n;

put_str("OK TH="); put_u32(n); put_nl();

} else {

put_str("ERR range 1100-5000"); put_nl();

}

} else if (!strcmp(line, "INT")) {

if (parse_u32(arg, &n) && n >= 1u && n <= 60u) {

s_report_s = (uint8_t)n;

s_rep_cnt = 0;

put_str("OK INT="); put_u32(n); put_nl();

} else {

put_str("ERR range 1-60"); put_nl();

}

} else if (!strcmp(line, "MUTE") || !strcmp(line, "UNMUTE")) {

s_muted = (line[0] == 'M');

outputs_apply();

put_str("OK MUTE="); put_u32(s_muted ? 1u : 0u); put_nl();

} else if (!strcmp(line, "SIM")) {

if (!strcmp(arg, "ADC"))       { s_src = SRC_ADC;  put_str("OK SIM=ADC"); put_nl(); }

else if (!strcmp(arg, "FAIL")) { s_src = SRC_FAIL; put_str("OK SIM=FAIL"); put_nl(); }

else if (parse_u32(arg, &n) && n >= 400u && n <= 5000u) {

s_src = SRC_PPM; s_sim_ppm = (uint16_t)n;

put_str("OK SIM="); put_u32(n); put_nl();

} else {

put_str("ERR range 400-5000"); put_nl();

}

} else {

put_str("ERR unknown cmd (try HELP)"); put_nl();

}

}

/* Collects characters into a line; executes it when Enter arrives. */

static void line_input(uint8_t c)

{

if (c == '\r' || c == '\n') {

if (s_line_discard) { s_line_discard = false; }

else if (s_line_len > 0) { s_line[s_line_len] = '\0'; cmd_exec(s_line); }

s_line_len = 0;

return;

}

if (s_line_discard) return;

if (s_line_len >= LINE_MAX_LEN) {           /* too long: drop whole line    */

put_str("ERR line too long"); put_nl();

s_line_discard = true;

s_line_len = 0;

return;

}

s_line[s_line_len++] = (char)c;

}

/* ---- 3.10 Init and scheduler ------------------------------------------- */

static void app_init(void)

{

s_blink = false;

outputs_apply();                            /* start with green LED on      */

put_str("HomeSense CO2 node v" FW_VERSION " READY (type HELP)");

put_nl();

}

/* One pass of the main loop: takes snapshots of the ISR flags, then runs the

* tasks that are due. Called continuously by main(), or once per simulated

* millisecond by the PC test harness.                                       */

static void app_step(void)

{

uint8_t a, b, c, ch, st;

st = hal_irq_save();                        /* read-and-clear flags safely  */

a = f_10ms; b = f_250ms; c = f_1s;

f_10ms = 0; f_250ms = 0; f_1s = 0;

hal_irq_restore(st);

while (rx_get(&ch)) line_input(ch);         /* handle UART commands         */

if (a) button_task();

if (b) { s_blink = !s_blink; outputs_apply(); }

if (c) sample_task();

}

/* =============================================================================

* SECTION 4 - HAL FOR REAL HARDWARE: ATmega328P (Arduino Uno, 16 MHz)

*   Pin map: D8..D10 = PB0..PB2 LEDs, D11 = PB3 buzzer, D2 = PD2 button,

*            A0 = ADC0 potentiometer, UART0 = USB serial at 9600 baud.

* ========================================================================== */

#ifdef TARGET_AVR

static void hal_init(void)

{

DDRB  |= (1 << PB0) | (1 << PB1) | (1 << PB2) | (1 << PB3); /* outputs      */

DDRD  &= (uint8_t)~(1 << PD2);                              /* button input */

PORTD |= (1 << PD2);                                        /* pull-up on   */

/* Timer1, CTC mode, prescaler 64: 16 MHz / 64 = 250 kHz; 250 counts = 1 ms */

TCCR1A = 0;

TCCR1B = (1 << WGM12) | (1 << CS11) | (1 << CS10);

OCR1A  = 249;

TIMSK1 = (1 << OCIE1A);

/* UART0: 9600 baud, 8N1, RX interrupt on. UBRR = F_CPU/16/9600 - 1 = 103 */

UBRR0H = 0;

UBRR0L = (uint8_t)(F_CPU / 16UL / 9600UL - 1UL);

UCSR0B = (1 << RXEN0) | (1 << TXEN0) | (1 << RXCIE0);

UCSR0C = (1 << UCSZ01) | (1 << UCSZ00);

/* ADC: AVcc reference, channel 0, prescaler 128 (125 kHz ADC clock) */

ADMUX  = (1 << REFS0);

ADCSRA = (1 << ADEN) | (1 << ADPS2) | (1 << ADPS1) | (1 << ADPS0);

set_sleep_mode(SLEEP_MODE_IDLE);

}

static void hal_led_set(uint8_t mask)

{

PORTB = (uint8_t)((PORTB & ~0x07u) | (mask & 0x07u));

}

static void hal_buzzer(bool on)

{

if (on) PORTB |= (1 << PB3); else PORTB &= (uint8_t)~(1 << PB3);

}

static int16_t hal_adc_read(void)

{

ADCSRA |= (1 << ADSC);                      /* start conversion             */

while (ADCSRA & (1 << ADSC)) { }            /* wait (about 104 us)          */

return (int16_t)ADC;

}

static bool    hal_button_pressed(void) { return !(PIND & (1 << PD2)); }

static void    hal_uart_putc(char c)    { while (!(UCSR0A & (1 << UDRE0))) { } UDR0 = (uint8_t)c; }

static uint8_t hal_irq_save(void)       { uint8_t s = SREG; cli(); return s; }

static void    hal_irq_restore(uint8_t s) { SREG = s; }

ISR(TIMER1_COMPA_vect) { timer_tick_isr(); }

ISR(USART_RX_vect)     { uart_rx_isr(UDR0); }

int main(void)

{

hal_init();

app_init();

sei();                                      /* enable interrupts            */

for (;;) {

app_step();

sleep_mode();                           /* idle until next interrupt    */

}

}

#endif /* TARGET_AVR */

/* =============================================================================

* SECTION 5 - HAL FOR PC SIMULATION + AUTOMATED SELF-TESTS

*   The "hardware" is plain variables. A test harness plays the role of the

*   timer (calling timer_tick_isr() once per simulated millisecond) and of the

*   UART receiver (calling uart_rx_isr() for each typed byte).

* ========================================================================== */

#ifdef HOST_SIM

static uint8_t  sim_led;                        /* current LED mask             */

static bool     sim_buz;

static int16_t  sim_adc = 512;                  /* simulated potentiometer      */

static bool     sim_btn;                        /* simulated button level       */

static uint32_t sim_ms;                         /* simulated time               */

static bool     sim_trace, sim_echo = true, sim_bol = true;

static char     cap[16384];                     /* copy of everything sent      */

static size_t   caplen;

/* hardware activity counters used by the tests (cleared by hw_reset()) */

static uint8_t  led_or;                         /* OR of every mask shown       */

static uint16_t led_zero, buz_on;               /* times LED all-off / buzzer on*/

static void hal_init(void) { }

static uint8_t hal_irq_save(void) { return 0; }

static void hal_irq_restore(uint8_t s) { (void)s; }

static int16_t hal_adc_read(void) { return sim_adc; }

static bool hal_button_pressed(void) { return sim_btn; }

static void hal_led_set(uint8_t mask)

{

if (sim_trace && mask != sim_led)

printf("  HW> t=%6lums LED G=%d Y=%d R=%d\n", (unsigned long)sim_ms,

!!(mask & LED_G), !!(mask & LED_Y), !!(mask & LED_R));

sim_led = mask;

led_or |= mask;

if (mask == 0) led_zero++;

}

static void hal_buzzer(bool on)

{

if (sim_trace && on != sim_buz)

printf("  HW> t=%6lums BUZZER %s\n", (unsigned long)sim_ms, on ? "ON" : "OFF");

sim_buz = on;

if (on) buz_on++;

}

static void hal_uart_putc(char c)

{

if (caplen < sizeof cap - 1) { cap[caplen++] = c; cap[caplen] = '\0'; }

if (sim_echo && c != '\r') {

if (sim_bol) fputs("  UART> ", stdout);

putchar(c);

sim_bol = (c == '\n');

}

}

/* ---- test harness helpers ---------------------------------------------- */

static void run_ms(uint32_t ms)                 /* advance simulated time       */

{

while (ms--) { sim_ms++; timer_tick_isr(); app_step(); }

}

static void uart_send(const char *s)            /* type text on the "terminal"  */

{

printf("  YOU > %s\n", s);

while (*s) uart_rx_isr((uint8_t)*s++);

uart_rx_isr('\n');

app_step();

}

static void cap_clear(void) { caplen = 0; cap[0] = '\0'; }

static bool cap_has(const char *s) { return strstr(cap, s) != NULL; }

static int  cap_count(const char *s)

{

int n = 0;

const char *p = cap;

while ((p = strstr(p, s)) != NULL) { n++; p += strlen(s); }

return n;

}

static void hw_reset(void) { led_or = 0; led_zero = 0; buz_on = 0; }

static int tests, passed;

#define CHECK(cond, name) do { tests++; if (cond) { passed++; \

printf("[PASS] %s\n", name); } else { \

printf("[FAIL] %s   (line %d)\n", name, __LINE__); } } while (0)

static int run_tests(void)

{

/* -- T0: boot ------------------------------------------------------- */

printf("\n== T0 Boot banner and initial outputs ==\n");

CHECK(cap_has("READY"), "boot banner printed over UART");

CHECK(sim_led == LED_G, "green LED on after boot");

/* -- T1: timer interrupt time base ---------------------------------- */

printf("\n== T1 Timer interrupt time base ==\n");

cap_clear();

run_ms(999);

CHECK(s_uptime == 0, "no 1 s tick after 999 ms");

run_ms(1);

CHECK(s_uptime == 1, "exactly one 1 s tick after 1000 ms");

run_ms(9000);

CHECK(s_uptime == 10, "10 ticks after 10 s");

/* -- T2: ADC to ppm conversion -------------------------------------- */

printf("\n== T2 ADC to ppm conversion ==\n");

CHECK(adc_to_ppm(0) == 400,    "ADC 0    -> 400 ppm");

CHECK(adc_to_ppm(512) == 1200, "ADC 512  -> 1200 ppm");

CHECK(adc_to_ppm(1023) == 2000,"ADC 1023 -> 2000 ppm");

/* -- T3: GOOD level --------------------------------------------------- */

printf("\n== T3 Good air: green LED, periodic report ==\n");

cap_clear();

sim_adc = 100;                              /* 556 ppm */

run_ms(5000);

CHECK(s_level == LVL_GOOD && sim_led == LED_G, "556 ppm -> GOOD, green LED solid");

CHECK(!s_alert, "no alert at 556 ppm");

CHECK(cap_has("RPT CO2=556ppm"), "periodic report shows filtered value");

/* -- T4: FAIR level --------------------------------------------------- */

printf("\n== T4 Fair air: yellow LED ==\n");

sim_adc = 400;                              /* 1025 ppm */

run_ms(5000);

CHECK(s_level == LVL_FAIR && sim_led == LED_Y, "1025 ppm -> FAIR, yellow LED solid");

/* -- T5: alert persistence ------------------------------------------- */

printf("\n== T5 Alert needs 3 consecutive samples above threshold ==\n");

cap_clear();

sim_adc = 900;                              /* 1807 ppm */

run_ms(4000);

CHECK(!s_alert, "no alert yet after 4 s (filter ramping, persistence counting)");

run_ms(1000);

CHECK(s_alert, "alert active after 5 s");

CHECK(cap_has("EVT ALERT ON"), "ALERT ON event printed");

hw_reset();

run_ms(1000);

CHECK(led_or == LED_R && led_zero > 0, "red LED blinks (on and off) during alert");

CHECK(buz_on > 0, "buzzer sounds during alert");

/* -- T6: mute command ------------------------------------------------- */

printf("\n== T6 MUTE / UNMUTE over UART ==\n");

cap_clear();

uart_send("MUTE");

CHECK(cap_has("OK MUTE=1"), "MUTE acknowledged");

hw_reset();

run_ms(1000);

CHECK(buz_on == 0 && led_zero > 0, "buzzer silent but red LED still blinks");

uart_send("unmute");                        /* lower case on purpose */

hw_reset();

run_ms(1000);

CHECK(buz_on > 0, "UNMUTE restores buzzer (command is case-insensitive)");

/* -- T7: button debounce --------------------------------------------- */

printf("\n== T7 Button debounce (contact bounce must count once) ==\n");

cap_clear();

sim_btn = true;  run_ms(10);                /* bounce: 10 ms pulses */

sim_btn = false; run_ms(10);

sim_btn = true;  run_ms(10);

sim_btn = false; run_ms(10);

CHECK(!s_muted && cap_count("EVT BUTTON") == 0, "short bounce pulses are ignored (no event)");

sim_btn = true;  run_ms(100);               /* real press */

CHECK(s_muted, "stable press toggles mute ON");

sim_btn = false; run_ms(100);               /* release */

CHECK(s_muted && cap_count("EVT BUTTON") == 1, "release does nothing; exactly one event");

sim_btn = true;  run_ms(100);

sim_btn = false; run_ms(100);

CHECK(!s_muted, "second press toggles mute OFF");

/* -- T8: hysteresis --------------------------------------------------- */

printf("\n== T8 Hysteresis: alert holds until 50 ppm below threshold ==\n");

cap_clear();

sim_adc = 691;                              /* 1480 ppm: below TH but inside hysteresis band */

run_ms(6000);

uart_send("STATUS");

CHECK(cap_has("CO2=1480ppm") && cap_has("ALERT=1"), "1480 ppm keeps the alert active");

sim_adc = 600;                              /* 1338 ppm */

cap_clear();

run_ms(6000);

CHECK(!s_alert && cap_has("EVT ALERT OFF"), "1338 ppm clears the alert");

CHECK(sim_led == LED_Y, "LED returns to solid yellow");

/* -- T9: UART commands ----------------------------------------------- */

printf("\n== T9 UART command interpreter ==\n");

cap_clear(); uart_send("TH 2000");

CHECK(cap_has("OK TH=2000") && s_th == 2000, "TH 2000 accepted");

cap_clear(); uart_send("TH 50");

CHECK(cap_has("ERR range") && s_th == 2000, "TH 50 rejected, value unchanged");

cap_clear(); uart_send("TH abc");

CHECK(cap_has("ERR range"), "non-numeric argument rejected");

uart_send("TH 1500");

cap_clear(); uart_send("INT 2");

CHECK(cap_has("OK INT=2"), "INT 2 accepted");

cap_clear();

run_ms(6000);

CHECK(cap_count("RPT") == 3, "6 s at 2 s interval -> exactly 3 reports");

uart_send("INT 5");

cap_clear(); uart_send("FOO");

CHECK(cap_has("ERR unknown"), "unknown command reported");

cap_clear(); uart_send("HELP");

CHECK(cap_has("CMDS:"), "HELP lists the commands");

cap_clear();

uart_send("THISLINEISFARTOOLONGTOBEACOMMAND");

CHECK(cap_has("ERR line too long"), "over-long line rejected safely");

cap_clear(); uart_send("STATUS");

CHECK(cap_has("CO2="), "interpreter still works after the long line");

/* -- T10: SIM override ------------------------------------------------ */

printf("\n== T10 SIM test mode ==\n");

cap_clear(); uart_send("SIM 3000");

CHECK(cap_has("OK SIM=3000"), "SIM 3000 accepted");

run_ms(8000);

CHECK(s_alert, "forced 3000 ppm raises the alert");

cap_clear(); uart_send("SIM 100");

CHECK(cap_has("ERR range"), "SIM 100 rejected (below 400)");

uart_send("SIM ADC");

run_ms(8000);

CHECK(!s_alert && s_src == SRC_ADC, "back on ADC input, alert cleared");

/* -- T11: sensor fault ------------------------------------------------ */

printf("\n== T11 Sensor fault detection and recovery ==\n");

cap_clear(); uart_send("SIM FAIL");

hw_reset();

run_ms(3000);

CHECK(s_fault && cap_has("EVT SENSOR FAULT"), "3 failed reads declare a FAULT");

CHECK((led_or & 7u) == 7u, "all three LEDs blink together during fault");

uart_send("SIM ADC");

cap_clear();

run_ms(2000);

CHECK(!s_fault && cap_has("EVT SENSOR OK"), "fault clears when the sensor returns");

/* -- T12: UART buffer overflow --------------------------------------- */

printf("\n== T12 UART ring buffer overflow handling ==\n");

{

int i;

for (i = 0; i < 100; i++) uart_rx_isr('A');   /* burst, main loop not run */

CHECK(rx_overflow > 0, "overflowing bytes are counted and dropped, no crash");

cap_clear();

app_step();                                    /* drain */

uart_rx_isr('\n');

app_step();

uart_send("STATUS");

CHECK(cap_has("CO2="), "firmware recovers and answers STATUS afterwards");

}

printf("\n==============================================\n");

printf(" RESULT: %d / %d checks passed\n", passed, tests);

printf("==============================================\n");

return (passed == tests) ? 0 : 1;

}

/* Interactive mode: lets the user drive the simulated device by hand. */

static void interactive(void)

{

char ln[96];

size_t n;

puts("Interactive mode. Text = UART command. @run N | @adc N | @btn 0/1 | @quit");

while (fgets(ln, sizeof ln, stdin)) {

n = strlen(ln);

while (n && (ln[n - 1] == '\n' || ln[n - 1] == '\r')) ln[--n] = '\0';

if (!strncmp(ln, "@run ", 5))       run_ms((uint32_t)strtoul(ln + 5, NULL, 10));

else if (!strncmp(ln, "@adc ", 5))  sim_adc = (int16_t)strtol(ln + 5, NULL, 10);

else if (!strncmp(ln, "@btn ", 5))  sim_btn = (ln[5] == '1');

else if (!strcmp(ln, "@quit"))      break;

else if (n)                         uart_send(ln);

}

}

int main(int argc, char **argv)

{

int i;

bool inter = false;

for (i = 1; i < argc; i++) {

if (!strcmp(argv[i], "--trace"))       sim_trace = true;

if (!strcmp(argv[i], "--interactive")) inter = true;

}

hal_init();

app_init();

if (inter) { interactive(); return 0; }

return run_tests();

}

#endif /* HOST_SIM */