HomeSense Node - Firmware Test Log (Week 2)

Source file: homesense_test_log.txt  |  Author: Mangesh Biradar  |  Yuva Intern - Embedded Systems

HOMESENSE NODE - WEEK 2 FIRMWARE TEST LOG

File under test : homesense_firmware.c

Test method     : PC simulation (HOST_SIM back-end): simulated 1 ms timer interrupt, simulated UART RX interrupt, simulated ADC/button

Environment     : gcc (Ubuntu 13.3.0-6ubuntu2~24.04.1) 13.3.0

### 1. BUILD (strict warnings)

$ gcc -std=c99 -Wall -Wextra -pedantic -DHOST_SIM -o homesense_sim homesense_firmware.c

(exit code 0 - no warnings, no errors)

### 2. AUTOMATED TESTS  (UART> = text the device printed, YOU > = text typed by the tester)

$ ./homesense_sim

UART> HomeSense CO2 node v1.0 READY (type HELP)

== T0 Boot banner and initial outputs ==

[PASS] boot banner printed over UART

[PASS] green LED on after boot

== T1 Timer interrupt time base ==

[PASS] no 1 s tick after 999 ms

[PASS] exactly one 1 s tick after 1000 ms

UART> [t=5s] RPT CO2=1200ppm inst=1200 LVL=FAIR

UART> [t=10s] RPT CO2=1200ppm inst=1200 LVL=FAIR

[PASS] 10 ticks after 10 s

== T2 ADC to ppm conversion ==

[PASS] ADC 0    -> 400 ppm

[PASS] ADC 512  -> 1200 ppm

[PASS] ADC 1023 -> 2000 ppm

== T3 Good air: green LED, periodic report ==

UART> [t=15s] RPT CO2=556ppm inst=556 LVL=GOOD

[PASS] 556 ppm -> GOOD, green LED solid

[PASS] no alert at 556 ppm

[PASS] periodic report shows filtered value

== T4 Fair air: yellow LED ==

UART> [t=20s] RPT CO2=1025ppm inst=1025 LVL=FAIR

[PASS] 1025 ppm -> FAIR, yellow LED solid

== T5 Alert needs 3 consecutive samples above threshold ==

[PASS] no alert yet after 4 s (filter ramping, persistence counting)

UART> [t=25s] EVT ALERT ON CO2=1807ppm

UART> [t=25s] RPT CO2=1807ppm inst=1807 LVL=POOR

[PASS] alert active after 5 s

[PASS] ALERT ON event printed

[PASS] red LED blinks (on and off) during alert

[PASS] buzzer sounds during alert

== T6 MUTE / UNMUTE over UART ==

YOU > MUTE

UART> OK MUTE=1

[PASS] MUTE acknowledged

[PASS] buzzer silent but red LED still blinks

YOU > unmute

UART> OK MUTE=0

[PASS] UNMUTE restores buzzer (command is case-insensitive)

== T7 Button debounce (contact bounce must count once) ==

[PASS] short bounce pulses are ignored (no event)

UART> [t=28s] EVT BUTTON MUTE=1

[PASS] stable press toggles mute ON

[PASS] release does nothing; exactly one event

UART> [t=28s] EVT BUTTON MUTE=0

[PASS] second press toggles mute OFF

== T8 Hysteresis: alert holds until 50 ppm below threshold ==

UART> [t=30s] RPT CO2=1643ppm inst=1480 LVL=POOR

YOU > STATUS

UART> CO2=1480ppm LVL=POOR ALERT=1 TH=1500 INT=5s MUTE=0 SRC=ADC FAULT=0

[PASS] 1480 ppm keeps the alert active

UART> [t=35s] EVT ALERT OFF CO2=1444ppm

UART> [t=35s] RPT CO2=1444ppm inst=1338 LVL=FAIR

UART> [t=40s] RPT CO2=1338ppm inst=1338 LVL=FAIR

[PASS] 1338 ppm clears the alert

[PASS] LED returns to solid yellow

== T9 UART command interpreter ==

YOU > TH 2000

UART> OK TH=2000

[PASS] TH 2000 accepted

YOU > TH 50

UART> ERR range 1100-5000

[PASS] TH 50 rejected, value unchanged

YOU > TH abc

UART> ERR range 1100-5000

[PASS] non-numeric argument rejected

YOU > TH 1500

UART> OK TH=1500

YOU > INT 2

UART> OK INT=2

[PASS] INT 2 accepted

UART> [t=42s] RPT CO2=1338ppm inst=1338 LVL=FAIR

UART> [t=44s] RPT CO2=1338ppm inst=1338 LVL=FAIR

UART> [t=46s] RPT CO2=1338ppm inst=1338 LVL=FAIR

[PASS] 6 s at 2 s interval -> exactly 3 reports

YOU > INT 5

UART> OK INT=5

YOU > FOO

UART> ERR unknown cmd (try HELP)

[PASS] unknown command reported

YOU > HELP

UART> CMDS: HELP STATUS TH <ppm> INT <s> MUTE UNMUTE SIM <ppm|ADC|FAIL>

[PASS] HELP lists the commands

YOU > THISLINEISFARTOOLONGTOBEACOMMAND

UART> ERR line too long

[PASS] over-long line rejected safely

YOU > STATUS

UART> CO2=1338ppm LVL=FAIR ALERT=0 TH=1500 INT=5s MUTE=0 SRC=ADC FAULT=0

[PASS] interpreter still works after the long line

== T10 SIM test mode ==

YOU > SIM 3000

UART> OK SIM=3000

[PASS] SIM 3000 accepted

UART> [t=49s] EVT ALERT ON CO2=2584ppm

UART> [t=51s] RPT CO2=3000ppm inst=3000 LVL=POOR

[PASS] forced 3000 ppm raises the alert

YOU > SIM 100

UART> ERR range 400-5000

[PASS] SIM 100 rejected (below 400)

YOU > SIM ADC

UART> OK SIM=ADC

UART> [t=56s] RPT CO2=2169ppm inst=1338 LVL=POOR

UART> [t=58s] EVT ALERT OFF CO2=1338ppm

UART> [t=61s] RPT CO2=1338ppm inst=1338 LVL=FAIR

[PASS] back on ADC input, alert cleared

== T11 Sensor fault detection and recovery ==

YOU > SIM FAIL

UART> OK SIM=FAIL

UART> [t=65s] EVT SENSOR FAULT CO2=1338ppm

[PASS] 3 failed reads declare a FAULT

[PASS] all three LEDs blink together during fault

YOU > SIM ADC

UART> OK SIM=ADC

UART> [t=66s] EVT SENSOR OK CO2=1338ppm

UART> [t=66s] RPT CO2=1338ppm inst=1338 LVL=FAIR

[PASS] fault clears when the sensor returns

== T12 UART ring buffer overflow handling ==

[PASS] overflowing bytes are counted and dropped, no crash

UART> ERR line too long

YOU > STATUS

UART> CO2=1338ppm LVL=FAIR ALERT=0 TH=1500 INT=5s MUTE=0 SRC=ADC FAULT=0

[PASS] firmware recovers and answers STATUS afterwards

==============================================

RESULT: 45 / 45 checks passed

==============================================

(exit code 0)

### 3. MEMORY / UNDEFINED-BEHAVIOUR CHECK

$ gcc -std=c99 -g -fsanitize=address,undefined -DHOST_SIM -o hs_asan homesense_firmware.c && ./hs_asan

exit code 0, sanitizer messages: 0  (0 = clean)

### 4. DO THE TESTS REALLY DETECT BUGS?  (bug injection)

Injected bug: PERSIST_SAMPLES 3 -> 1 (alert too eager)

[FAIL] no alert yet after 4 s (filter ramping, persistence counting)   (line 733)

RESULT: 44 / 45 checks passed

Injected bug: HYST_PPM 50 -> 0 (no hysteresis)

[FAIL] 1480 ppm keeps the alert active   (line 777)

[FAIL] 1338 ppm clears the alert   (line 781)

RESULT: 43 / 45 checks passed

Injected bug: DEBOUNCE_TICKS 3 -> 0 (no debounce)

[FAIL] short bounce pulses are ignored (no event)   (line 762)

[FAIL] release does nothing; exactly one event   (line 766)

RESULT: 43 / 45 checks passed

Injected bug: ADC scale 1600 -> 1500 (wrong conversion)

[FAIL] ADC 512  -> 1200 ppm   (line 710)

[FAIL] ADC 1023 -> 2000 ppm   (line 711)

[FAIL] periodic report shows filtered value   (line 720)

[FAIL] 1025 ppm -> FAIR, yellow LED solid   (line 726)

[FAIL] 1480 ppm keeps the alert active   (line 777)

[FAIL] 1338 ppm clears the alert   (line 781)

RESULT: 39 / 45 checks passed

### 5. HARDWARE TRACE EXAMPLE (LED/buzzer changes, first events)

$ ./homesense_sim --trace

HW> t=     0ms LED G=1 Y=0 R=0

HW> t=  1000ms LED G=0 Y=1 R=0

HW> t= 12000ms LED G=1 Y=0 R=0

HW> t= 19000ms LED G=0 Y=1 R=0

HW> t= 25000ms LED G=0 Y=0 R=0

HW> t= 25250ms LED G=0 Y=0 R=1

HW> t= 25250ms BUZZER ON

HW> t= 25500ms LED G=0 Y=0 R=0

HW> t= 25500ms BUZZER OFF

HW> t= 25750ms LED G=0 Y=0 R=1

HW> t= 25750ms BUZZER ON

HW> t= 26000ms LED G=0 Y=0 R=0

HW> t= 26000ms BUZZER OFF

HW> t= 26250ms LED G=0 Y=0 R=1